| NAME | PATRICK WALSH |
| --- | --- |
| I.D. | 16177789 |
| SUPERVISOR | IAN CLANCY |
| COURSE | B.Sc. APPLIED PHYSICS |
| YEAR | FOURTH YEAR |
| PROJECT TITLE | THE DYNAMICS OF FREELY SPINNING OBJECTS ON FRICTIONAL SURFACES |
| DATE | 24th April 2020 |

Contents

1.	Introduction	3

1.1.	The dynamics of spinning objects	3

1.2.	The Equations of Motion	5

1.3.	Solving the ODEs	8

1.4.	The ODE45 method	9

1.5.	Variation of initial conditions	11

1.6.	Lyapunov exponents	12

1.7.	The edge of chaos	13

1.8.	Project aims	13

2.	Literature Review	14

2.1.	The inertia tensor	14

2.2.	Euler vectors/Euler angles	16

2.2.1.	Terminal retrograde turn of rolling rings	16

2.2.2.	Rolling Friction and Energy Dissipation in a spinning disk	17

3.	Simulated Model	20

4.	Measurements, Results and Analysis	25

4.1.	Effect of initial θ and ϕ on the behaviour along the trajectory	25

4.2.	Effect of initial ψ on the behaviour along the trajectory	28

4.3.	Lyapunov exponents due to variation in initial nutation	31

4.4.	Variation of the drag between the disk and the surface	34

4.5.	Calculating the Lyapunov exponents vs θ0	41

5.	Interpretation and Discussion	47

5.1.	The path of the COM in the xy-plane	47

5.2.	The evolution of the disk’s angular velocity	48

5.3.	The evolution of the disk’s nutation	50

5.4.	The approximated frictional model	51

5.5.	The error prone Lyapunov exponent	52

5.6.	Chaotic nature of the system relative to the initial nutation	53

5.7.	Why do these local extremes appear?	54

6.	Conclusions	55

7.	Appendix	57

8.	Contribution Checklist	59

9.	References	60

When a disk is spun on a frictional surface, the trajectory of the disk is extremely sensitive to initial conditions and, under the right circumstances, can display chaotic behaviour. This paper details the use of MATLAB to develop a model which can simulate the motion of such a spinning disk system, explores the most influential initial conditions of the disk in terms of changing the behaviour of the disk in motion, studies the exact relationship between each of these initial conditions and the trajectory of the centre of mass and ultimately analyses the effect of possibly the most affective initial condition on the amount of chaos exhibited by the spinning disk system using the theory of Lyapunov exponents. The results detailed show a relation between the level of chaos exhibited by the system and the level of retrograde experienced by the centre of mass motion with the sensitivity increasing dramatically as the initial condition approaches its maximum value.

# Introduction

## The dynamics of spinning objects

When modelling a spinning disk, the concept of Euler’s disk is used define the disks motion and its degrees of freedom. The motion of a spinning/rolling disk can be modelled in terms of Euler vectors and Euler angles. Consider a disk as illustrated in Figure 1.1: Graphic of the relevant vectors and angles of Euler’s disk. [1] (Detailed also in Literature review)

Figure 1.1: Graphic of the relevant vectors and angles of Euler’s disk

The diagram above helps to visualise the physical meaning of all of the Euler vectors and angles. The vectors e1, e2 and e3 represent the Euler vectors in the frame of the disk. e1 always points parallel to the surface, e3 is perpendicular to e1 in the plane of the disk (pointing away from the point of contact with the surface) and e2 points through the axis of symmetry of the disk, perpendicular to both e1 and e3.

The Euler angles θ, ψ and ϕ represent the nutation (tilt), the revolution (about e2) and the precession of the disk about the vertical.

Outside of the rotating frame of the disk, an inertial frame is defined which helps to model the motion of the centre of mass of the disk and the contact point A between the disk and the flat surface. The basis for this frame is taken as i,  j, k and the co-ordinates used to describe the position of a point on the disk in the inertial frame are (x, y, z).

Using this model, the disk has six degrees of freedom; (x, y, z, θ, ϕ, ψ). These quantities are related to one another through second order ordinary differential equations. The notation involved in these ODEs tends to change based on which piece of literature one is looking at. Some relevant symbols are defined as follows for the entirety of this study:

- a≡disk radius

- h≡disk width

- m≡disk mass

- g≡gravitational acceleration

- Ci≡drag coefficients

## The Equations of Motion

Many of these ODEs can be easily expressed in matrix form and are all inter-related. The ultimate goal is to solve for the six degrees of freedom x,  y,  z,  θ,  ϕ,  ψ. To achieve this, we develop three sets of ODEs in terms of nine main quantities:

- The three components of the disk’s angular velocity ω1,  ω2,  ω3

- The three Euler angles θ,  ϕ,  ψ

- The three position co-ordinates of the disk’s centre of mass x,  y,  z

1. The three components of the disk’s angular velocity. The angular velocity vector ω can be found by solving the ODE in Equation 1.1 below. The necessary components of this equation are defined also:

J .ω=-Ω×LG-m rG×Ω×vc+mgasinθ-h2cosθe1+TDrag Equation 1.1: First order differential equation of the disk angular velocity

I=m3a2+h212000a220003a2+h212, J=I+mh24+a2000a2-ah20-ah2h24 Equation 1.2: Inertia tensor of disk

ω=θϕsinθ+ψϕcosθ Equation 1.3: Angular velocity of disk

Ω=ω-ψe2=θϕsinθϕcosθ Equation 1.4: Angular velocity of moving frame

LG=I.ω Equation 1.5: Angular momentum of disk wrt contact point

rG=0h/20 Equation 1.6: Position of center of mass wrt contact point

vC=rC=ω×rG Equation 1.7: Velocity of center of mass wrt fixed frame

TDrag=-Ji3Ciωiωi. ei Equation 1.8: Drag induced torque of disk

Once Equation 1.2 through Equation 1.8 are defined in MATLAB, Equation 1.1 can be defined. This is a first order ODE in ω. Equation 1.1 is expressed in terms of ω and is defined by its own function ‘Domega’. This equation is then manipulated to find an equation for the Euler angle derivatives:

2. The three Euler angles. One defines a matrix ‘o2DE’ to transform ω to θϕψ:

o2DE=10001-tanθ001cosθ Equation 1.9: Transformation matrix to produce the Euler angle ODEs from ω

DEuler=o2DE*omega Equation 1.10: Euler angle ODE set

This provides the second set of first order ODEs

3. The three position co-ordinates of the disk’s centre of mass. The position co-ordinates can be defined in a similar way as above using the Euler angles this time.

A function ‘Euler2Cartesian’ is defined which sets the value of three transformation functions:

R1α=1000cosαsinα0-sinαcosα R2α=cosα0-sinα010sinα0cosα R3α=cosαsinα0-sinαcosα0001 Equation 1.11: Transformation functions to convert from angular to cartesian form

Two functions are then defined; C2E and E2C to convert from cartesian to angular form and back. This makes it easier to convert from Euler to cartesian as one can solve the easier form of cartesian to angular and then invert the matrix to convert the other way:

C2E=R1θ*R3ϕ, E2C=inv(C2E)

R2 is not needed here as only θ and ϕ are needed to determine the cartesian position given a fixed radius, a. The centre of mass does not move as the revolution angle ψ changes.

The position ODE set is then expressed as:

Dposition=E2CEuler*vC Equation 1.12: Position co-ordinates ODE set.

This gives us three functions; Dposition, DEuler and Domega, each representing three ODEs. These functions can then be concatenated into one function of nine ODEs to be solved in parallel:

## Solving the ODEs

Solving these ODEs in MATLAB, one requires the use of an ODE solver. The one that will be used throughout this project is ODE45. The numerical method this is working behind the scenes when ODE45 is called. The method takes in the concatenated function [Dposition;DEuler;Domega] (func below) and solves the ODEs involved numerically. The Integration times and the initial values of the position, Euler angles and omega vector are also set.

[t, x] = ode45(@(t, x)ConcatenatedFunction, IntegrationTimes, [position; Euler0; omega0], opts);

The numerical method that the ODE45 method is based on is detailed below

## The ODE45 method

The working principal of ODE45 is based on the Runge-Kutta method. A Runge-Kutta method is a numerical analysis technique used to approximately solve an ordinary differential equation. They are used heavily in computation when finding analytical solutions can be challenging. These methods can be implicit or explicit methods; the RK4 method (which is generally referred to as ‘the runge-kutta method’) is an example of an explicit numerical method. There are infinitely many iterations of the Runge-Kutta method (1st order, 2nd order, …, nth order) each one more accurate than the last. Each time the order is increased, the accuracy increases by a smaller and smaller amount i.e. most of the accuracy of the method is achieved in the first few orders; similar to a Taylor series expansion. The ODE45 method is so named as it employs 4th and 5th order RK methods to solve the ODE. A 5th order RK method is used to solve the ODEs, then the process is repeated, this time using a 4th order RK method. The two results are then compared and if they are not sufficiently close, the time step used for the RK methods is narrowed and the process repeated. This process is repeated until the results of the 4th and 5th order RK methods are within a certain degree of closeness, at which point the solution is accepted as sufficiently accurate.

The general formulae for solving an explicit RK method is based on discrete iterations, and so a step size must be set. The iterative value of the function y is given by

yn+1=yn+hi=1sbiki Equation 1.13: Iterative formula for solving y from y

where

y=ft,y,  yt0=y0

The time step is then set to be h such that

tn+1=tn+h

The values of ki in Equation 1.13 are given by the general form [4]

k1=ftn, yn

k2=ftn+c2h, yn+ha21k1

k3=ftn+c3h, yn+ha31k1+a32k2

⋮

ks=ftn+csh, yn+has1k1+as2k2+…+as,s-1ks-1

Only first order equations can be solved by a RK4 or RK5 method, therefore if trying to solve a second order equation using ODE45, such an ODE would need to be expressed as two first order ODEs as in the following example:

x=x+x

becomes

x1=x2

x2=x2+x(1)

Here x1 represents x and x2 represents x.

This numerical method is encoded into ODE45 and solves the equations of motion of the spinning disk system creating a working model on which to run simulations. The model can therefore be used to answer some interesting questions such as how sensitive the disk is to each initial condition on which it depends.

## Variation of initial conditions

There are a number of parameters of the system which should influence the behaviour of the disk’s motion. These include;

- The disk’s radius, a

- The disk’s mass, m

- The disk’s thickness, h

- Acceleration due to gravity, g

- The initial nutation and initial rate of nutation, θ and θ

- The initial rate of precession, ϕ

The effect of these parameters on the motion of the disk can be analysed by running the model for two separate sets of initial conditions and watching the change in behaviour of the system. This can shed light on how chaotic the system is. However, this analysis is very qualitative and is worth far more if there is a quantitative way of analysing the degree of sensitivity of the system to each initial condition. This is where Lyapunov exponents come in and give an easily expressible numerical value to the level of chaos that the system exhibits.

## Lyapunov exponents

Figure 1.2: Difference in path of a chaotic system using different initial conditions (exaggerated).

The concept of Lyapunov exponents are used to describe the level of chaos exhibited by a system. The basic idea involves defining two varying initial positions for the system to run from. (These initial positions may be varied directly or indirectly by varying another parameter which influences the position). The initial positions can be defined as x0 and x0+ϵ and the difference dn after n iterations (can also be expressed as time, t) will diverge exponentially in a chaotic system according to the equation:

dt=ϵeλt Equation 1.14: Lyapunov’s formula

The λ in Equation 1.14 is referred to as the Lyapunov exponent. If λ is negative, the equation above implies that the difference will approach 0 and the points will converge (meaning the system is not chaotic). If, however it is positive then they will diverge. The magnitude of the positive Lyapunov exponent indicates how chaotic the system is; in other words, how much the paths will diverge in a given time interval from a tiny change in the initial conditions of the system.

The value of λ can be easily determined by taking the log of both sides of Equation 1.14:

lndn=lnϵ+λt Equation 1.15: Natural log of Lyapunov’s formula

A graph can easily be generated of lndn vs t and the initial slope of this graph can be taken as the Lyapunov exponent.

## The edge of chaos

This study of the effect of the initial conditions of the system on its tendency to behave chaotically can lead into a further analysis of how exactly the initial conditions need to be changed to incite chaos. From here, one can figure out the exact initial conditions at which chaos sets in, the point that is sometimes referred to as ‘the onset of chaos’ or ‘the edge of chaos’. This is beyond the scope of this project, however, this project along with the model used could serve as a foundation for this type of study.

## Project aims

The aims of this project are to model the trajectory of the centre of mass of the spinning disk by looking at the physical path it takes, the variation in the disk’s angular velocity and the evolution of the angle of nutation over time, to study how this behaviour depends on the initial conditions and to analyse the Lyapunov exponent of the system and finally, to study the effect of friction on the trajectory of the disk.

# Literature Review

## The inertia tensor

The concept of the inertia tensor is useful in modelling the centre of mass of the spinning object as it moves in three dimensions. To build up the concept of the Inertia tensor we first look at some useful formulas. [5]

The rotational inertia of a system is given by

I=MR2

Consider a rigid body which is rotating with an angular velocity ω while also moving with a linear velocity V. The instantaneous velocity of a point α on the body can be expressed as

vα=V+ω×rα

T=12αmαV+ω×rα2

T=TT+TR

TT=Translational Kinetic Energy,  TR=Rotational Kinetic Energy

TT=12αmαV2,  TR=12αmαω×rα2

But, the square of a cross product can be expanded as

A×B2=A2B2-A∙B2

TR=12αmα(ω2rα2-ω∙rα2)

Let the position rα be represented by the co-ordinates

rα=xα,1,xα,2,xα,3

This results in the relationship for the rotational kinetic energy

TR=12ijωiωjαmαδijkxα,k2-xα,ixα,j

This allows us to define an Inertia term in i and j

TR=12ijIijωiωj,  Iij=αmαδijkxα,k2-xα,ixα,j

The 𝑇𝑅 term above takes the expected form

TR=12Iω2

The 9 values of Iij constitute I which is referred to as the inertia tensor. The tensor is needlessly cumbersome and less readable in its state defined by letting rα=xα,1,xα,2,xα,3 so this term is redefined as

rα=xα,yα,zα

This allows the inertia tensor to be written as

I=αmαrα2-xα2-αmαxαyα-αmαxαzα-αmαyαxααmαrα2-yα2-αmαyαzα-αmαzαxα-αmαzαyααmαrα2-zα2

The Iij term can be re-written for a body with a continuous matter distribution and mass density ρ [5]

ρ=ρr

Iij=Vρrδijkxk2-xixjdV,  dV=dx1dx2dx3

## Euler vectors/Euler angles

### Terminal retrograde turn of rolling rings

This paper by Jalali et al. [1] provided the framework for the model used in this project. The notation is detailed in the introduction and Figure 1.1 and the equations are reiterated:

I .ω-mrG×rG×ω=-Ω×LG-m rG×Ω×vc+mgasinθ-h2cosθe1 Equation 2.1: Angular velocity ODE in terms of Inertia

J .ω=-Ω×LG-m rG×Ω×vc+mgasinθ-h2cosθe1+TDrag (Equation 1.1)

J=I+mh24+a2000a2-ah20-ah2h24 (Equation 1.2)

TDrag=-Ji3Ciωiωi. ei (Equation 1.8)

The value of I used by Jalali and in Equation 1.2 can be found in the literature [2]:

I=m3a2+h212000a220003a2+h212 (Equation 1.2)

### Rolling Friction and Energy Dissipation in a spinning disk

Daolin Ma and their team [3] use the illustration in Figure 2 to visualise the disk:

Figure 2.1: Euler’s disk illustration (Ma et al.)

Note the notation difference as opposed to the paper by Jalali et al. where e2 and e3 are reversed as are ϕ and ψ. As stated earlier, the disk has six degrees of freedom; (x, y, z, ψ, θ, ϕ) which are related to one another through second order ODEs.

The radius is defined in this paper as r. The inertia tensor of the disk is defined as J with principal moments J1, J2 and J3.

J1=J2=14mr2+13mh2,  J3=12mr2

The absolute angular velocity of a point on the disk is Ω.

Ω=ω1ω2ω3

Keeping this in mind, the angular momentum LO about the centre of mass is written as

LO=J1ω1J1ω2J3ω3

We can define another co-ordinate frame at the point of contact A between the disk and the surface.

RA=m0n0k

Defining this allows us to introduce the resultant force FA and the net moment MA of the surface acting on the disk at the contact point A.

FA=FnFmFz, MA=MnMmMz

With this in mind the equations for the centre of mass are:

xyz=cosψsinψ0-sinψcosψ0001+ψrcosθ-hsinθ+ϕrθrsinθ+hcosθθsinθh-r Equation 2.2: Centre of mass position ODEs. [3]

And the angular equations:

I1θ-I2ψ2sinθcosθ+I3sinθ+mrhcosθψω3-mrhψ2sin2θ+mgrcosθ-hsinθ=0

I2ddtψsinθ+J1θψcosθ-J3θω3-mhrω3+mhψθrsinθ+hcosθ=0

I3ω3-mrhddtψsinθ-mrψθrsinθ+hcosθ=0 Equation 2.3: Euler angle ODEs. [3]

The quantities I1, I2 and I3 are based on the principle moments of inertia J:

I1I2I3=J1+mh2+r2J1+mh2J3+mr2

The energy of the system remains constant during the rolling motion due to a lack of dissipation:

E=12mx2+y2+z2+12J1ω12+J1ω22+J3ω32+mgz Equation 2.4: Constant energy term. [3]

The results of this study were not obtained by simulation, but rather by spinning a disk and recording the motion of the centre of mass with a high-speed camera. This means that the equations of motion in this study have not been used to generate the results in Figure 2.2: Nutation θ vs timeFigure 2.2 and Figure 2.3

Figure 2.2: Nutation θ vs time. [3]

Figure 2.3: Path of centre of mass in x-y plane. [3]

This is the paper on which this project’s model was originally based however, due to the complexity of the equations involved and the abundance of approximations and assumptions in the study, this model is impractical for simulating the results that Daolin Ma and his team obtained. The results obtained by Ma were not directly based on the equations of motion specified; these equations were derived separately as background theory. This model was abandoned and Jalali’s equations of motion were used instead as the equations involved were more fundamental and, from a modelling perspective, more practical.

# Simulated Model

All of the code used throughout the project is detailed below. It may also be found on GitHub. [6] The GitHub versions may include extra comments and clarification but are functionally the same.

The following code in Figure 3.1 models the motion of the spinning disk based on the equations of motion in the introduction.

Figure 3.1: MATLAB model of a spinning disk

Measuring the Lyapunov exponent involves taking the slope of the line of best fit of a suitable set of linear datapoints near the beginning of the plot. However, the system is extremely sensitive to the initial conditions and so the exact location of this set of datapoints varied erratically as theta was varied. This made the range of datapoints used for the LE calculation difficult to automate. The LE was therefore calculated manually for each value of Theta. A separate script (Figure 3.2) was used to record and plot the Lyapunov exponent λ at each value of θ. These Lyapunov exponent values were then normalised against the minimum value found for λθ.

The LE values are themselves highly erratic and are very sensitive to the exact range chosen to calculate them. Care was taken to ensure that the method for selecting the range was kept as consistent as possible for all angles. For this reason, the exact values calculated for the Lyapunov exponents may not be accurate and should not be taken as exact results but rather simply as an indication of the pattern of the relative magnitude of the Lyapunov exponent with Theta. The values of the Lyapunov exponents are therefore all normalised against the minimum value.

Figure 3.2: Lyapunov exponent plotter

# Measurements, Results and Analysis

## Effect of initial θ and ϕ on the behaviour along the trajectory

The trajectory of the centre of mass can be measured at any combination of initial values of θ and ϕ to a significantly different result. The same reasoning applies to the change in the three components of the angular velocity, ω1,  ω2 and ω3 as well as the variation of the angle of nutation over time. There are too many permutations to plot so to illustrate how these parameters depend on θ and ϕ, we can instead generate four plots with combinations of ‘high’ and ‘low’ initial values of θ and ϕ.

- θhigh=89°

- θlow=45°

- ϕhigh=80 revolutions t-1

- ϕlow=6.5 revolutions t-1

For Figure 4.1 through Figure 4.3 the initial values of ψ,  ϕ,  θ and ψ are all set to zero. The relevant initial conditions (θ and ϕ) are specified in the caption to each subplot. Some more relevant initial parameters of the disk include:

- m=1  (Disk mass)

- a=1 (Disk radius)

- h=2 (Disk height)

- C=0.03,  0.073,  0.185  (Drag coefficients) (It is worth noting that these drag coefficients imply a grain either in the disk or on the surface holding the disk due to the difference between Cx and Cy which would otherwise be equal)

| Figure 4.1a: θhigh, ϕhigh | Figure 4.1b: θhigh, ϕlow |
| --- | --- |
| Figure 4.1c: θlow, ϕhigh | Figure 4.1d: θlow, ϕlow |

Figure 4.1: Trajectories of centre of mass

| Figure 4.2a: θhigh, ϕhigh | Figure 4.2b: θhigh, ϕlow |
| --- | --- |
| Figure 4.2c: θlow, ϕhigh | Figure 4.2d: θlow, ϕlow |

Figure 4.2: Angular velocity spirals

| Figure 4.3a: θhigh, ϕhigh | Figure 4.3b: θhigh, ϕlow |
| --- | --- |
| Figure 4.3c: θlow, ϕhigh | Figure 4.3d: θlow, ϕlow |

Figure 4.3: Normalised nutation vs time

## Effect of initial ψ on the behaviour along the trajectory

The initial revolution rate ψ of the disk has been set to zero thus far. This can however have an interesting effect on the trajectory of the disk. The nutation was set to 89° and the same analysis was performed for combinations of ‘high’ and ‘low’ initial values of ψ and ϕ. (Figure 4.4 through Figure 4.6)

- ψhigh=ϕhigh=80 revolutions t-1

- ψlow=ϕlow=6.5 revolutions t-1

The values of m, a, h and C (mass, radius, height and drag coefficients) are identical to those in Section 5.1.

| Figure 4.4a: ψhigh, ϕhigh | Figure 4.4b: ψhigh, ϕlow |
| --- | --- |
| Figure 4.4c: ψlow, ϕhigh | Figure 4.4d: ψlow, ϕlow |

Figure 4.4: Trajectories of centre of mass with nonzero ψ0

| Figure 4.5a: ψhigh, ϕhigh | Figure 4.5b: ψhigh, ϕlow |
| --- | --- |
| Figure 4.5c: ψlow, ϕhigh | Figure 4.5d: ψlow, ϕlow |

Figure 4.5: Angular velocity spirals with nonzero ψ0

| Figure 4.6a: ψhigh, ϕhigh | Figure 4.6b: ψhigh, ϕlow |
| --- | --- |
| Figure 4.6c: ψlow, ϕhigh | Figure 4.6d: ψlow, ϕlow |

Figure 4.6: Normalised nutation vs time with nonzero ψ0

## Lyapunov exponents due to variation in initial nutation

Using the model and the background theory detailed previously (Equation 1.15), it is assumed that the absolute distance between the trajectories of the centre of mass at slightly different initial nutations increases roughly exponentially as time progresses. Using this theory, the log of this distance is plotted against time and a roughly straight line should result. The slope of this line should be equal to the Lyapunov exponent. These plots of ln⁡(dt) vs t can be generated for many different sets of slightly varying values of θ0, for instance:

- θ01=40°,  θ02=39.999°

- θ01=60°,  θ02=59.999°

This is carried out for all values of θ01 from 28° to 89° in increments of 1° with the slightly varied value θ02=θ01-0.001. The resultant plots are too numerous and the differences between them too gradual to include all of them in this report. The behaviour is instead summarised by showing the graphs in increments of 10° (i.e. 29°, 39°, …, 89°). The full list of images is also available on GitHub. [6]

A suitable range of points needs to be chosen for each plot in order to calculate the Lyapunov exponent at the corresponding nutation. This involves taking the slope of the line of best fit of a region near the beginning of the plot. The location of this range is highly subjective (as shown in Figure 4.7) and highly error prone. It is highly likely that the approximation of the distance between the centre of mass trajectories as exponentially increasing is inaccurate. However, the analysis can still be carried out for a relatively narrow range of datapoints to gain an understanding of the variation of λ with nutation. Due to the high sensitivity of the slope of the graph depending on the range of points chosen, the same region of the graph must be analysed for all values of θ. The range chosen for this project was the highly linear region immediately after the first local minimum e.g.

- θ0=29°→range begins @t≈0.04 (Figure 4.7a)

- θ0=69°→range begins @t≈0.07 (Figure 4.7e)

| Figure 4.7a: θ0=29° | Figure 4.7b: θ0=39° |
| --- | --- |
| Figure 4.7c: θ0=49° | Figure 4.7d: θ0=59° |
| Figure 4.7e: θ0=69° | Figure 4.7f: θ0=79° |
| Figure 4.7g: θ0=89° |  |

Figure 4.7: ln⁡(dt) vs t relationships at varying θ0

The values of m, a, h and C again remain unchanged

## Variation of the drag between the disk and the surface

The drag coefficients between the disk and the surface have so far remained unchanged throughout the project. These may be varied to determine the behaviour of the disk under various frictional conditions. The exact values selected for the drag coefficients, C are arbitrary (the values selected thus far throughout the project have been [0.03, 0.073, 0.185]) however one could determine from the equations involved what is the implicit unit of the drag coefficients (which assuming SI units would be unitless). The extent of this study is simply to show the behaviour of the disk with respect to the relative size of the drag coefficients rather than to measure the exact units involved in the drag coefficient. It is perhaps more productive to look at the drag in the xy-plane and along the z-axis separately. The initial nutation θ0=89° and the initial precession rate ϕ0=80.

First, we look at the drag coefficients in the xy-plane with Cz=0.185 as in previous sections

| Figure 4.8a: Cx=Cy=0.1 | Figure 4.8b: Cx=Cy=0.2 |
| --- | --- |
| Figure 4.8c: Cx=Cy=0.5 | Figure 4.8d: Cx=Cy=1 |

Figure 4.8: Trajectories of centre of mass with varying xy-plane drag

| Figure 4.9a: Cx=Cy=0.1 | Figure 4.9b: Cx=Cy=0.2 |
| --- | --- |
| Figure 4.9c: Cx=Cy=0.5 | Figure 4.9d: Cx=Cy=1 |

Figure 4.9: Angular velocity spirals with varying xy-plane drag

| Figure 4.10a: Cx=Cy=0.1 | Figure 4.10b: Cx=Cy=0.2 |
| --- | --- |
| Figure 4.10c: Cx=Cy=0.5 | Figure 4.10d: Cx=Cy=1 |

Figure 4.10: Evolution of normalised nutation with varying xy-plane drag

We can also analyse the effect of drag in the z-direction on the trajectory with Cx=Cy=0.1.

| Figure 4.11a: Cz=0.1 | Figure 4.11b: Cz=0.2 |
| --- | --- |
| Figure 4.11c: Cz=0.5 | Figure 4.11d: Cz=1 |

Figure 4.11: Trajectories of centre of mass with varying Cz

| Figure 4.12a: Cz=0.1 | Figure 4.12b: Cz=0.2 |
| --- | --- |
| Figure 4.12c: Cz=0.5 | Figure 4.12d: Cz=1 |

Figure 4.12: Angular velocity spirals with varying Cz

| Figure 4.13a: Cz=0.1 | Figure 4.13b: Cz=0.2 |
| --- | --- |
| Figure 4.13c: Cz=0.5 | Figure 4.13d: Cz=1 |

Figure 4.13: Evolution of normalised nutation with varying Cz

## Calculating the Lyapunov exponents vs θ0

This range of datapoints, as described earlier, was difficult to automate for all θ. The LE was therefore calculated manually (Table 7.1). The values are then expressed relative to λmin (Table 7.2). These data tables are illustrated in the appendix.

The data in Table 7.2 is illustrated in Error! Reference source not found.a and again in Error! Reference source not found.b but without the last eight datapoints to more easily visualise the relationship for the bulk of the plot.

The values of m, a, h and C again remain unchanged and the rate of precession was set unperturbed at ϕ=80 revolutions t-1 (No revolution ψ).

| Figure 4.14a: Entire relationship (smoothed plot) | Figure 4.14b: θ0=28°→81° (smoothed plot) |
| --- | --- |

Figure 4.14: Relative Lyapunov exponent vs initial nutation

The three notable points in Error! Reference source not found.b are the local minima at θ=30°, 66° and the local maximum at θ=45°. These points may be further analysed by looking at the trajectories of the centre of mass around θ=30°, 45°.

| Figure 4.15a: θ=31° tmax=6.99 | Figure 4.15b: θ=32° |
| --- | --- |
| Figure 4.15c: θ=33° | Figure 4.15d: θ=34° |

Figure 4.15: Centre of mass trajectories from t=0 to t=50 (unless otherwise specified) for θ→30°

| Figure 4.16a: θ=41° | Figure 4.16b: θ=42° |
| --- | --- |
| Figure 4.16c: θ=43° | Figure 4.16d: θ=44° |
| Figure 4.16e: θ=45° |  |
| Figure 4.16f: θ=46° | Figure 4.16g: θ=47° |
| Figure 4.16h θ=48° | Figure 4.16i: θ=49° |

Figure 4.16: Centre of mass trajectories from t=0 to t=50 for θ→45°

| Figure 4.17a: θ=62° | Figure 4.17b: θ=63° |
| --- | --- |
| Figure 4.17c: θ=64° | Figure 4.17d: θ=65° |

Figure 4.17: Centre of mass trajectories from t=0 to t=50 for θ→66°

# Interpretation and Discussion

## The path of the COM in the xy-plane

The effect of the initial conditions on the path of the centre of mass of the disk is illustrated in Figure 4.1, Figure 4.4, Figure 4.8 and Figure 4.11. Looking at the axes of the four plots in Figure 4.1, it is clear that the distance across which the centre of mass travels is maximised when the initial nutation and initial rate of precession are both high whereas when either θ0 or ϕ0 are low, the disk is not able to travel as far. It is also observed that the disk undergoes far less ‘orbiting’ behaviour when the initial nutation is low, displaying instead a more unstable ‘rocking’ motion indicated by the frequent looping of the centre of mass along the trajectory.

From Figure 4.4 it is clear that, again, the disk tends to travel significantly further when a high revolution rate ψ is combined with the regular precession motion (comparing 5.4a and 5.4b with 5.4c and 5.4d). A more potentially surprising result is the fact that the disk travelled significantly further when a high rate of revolution was combined with a low rate of precession as opposed to when both rates were high. While initially unintuitive, this makes sense as the high rate of precession can work to nullify the effect of the high revolution rate. When this precession is lessened however, the revolution tends to be more dominant which may cause the disk to roll a large distance before the precessive motion takes over.

Looking at Figure 4.8 and Figure 4.11, it is clear that the behaviour of the disk with changing Cx and Cy is highly different from the behaviour as Cz is changed. As the drag on the xy-plane is increased, the disk appears to undergo less initial orbiting and falls into the more linear motion towards the end of the trajectory sooner. However, as the drag along the z-axis is increased, the opposite effect is observed. The disk undergoes far more orbiting as the effect of Cz rises.

## The evolution of the disk’s angular velocity

The evolution of the angular velocity vectors are visualised in Figure 4.2, Figure 4.5, Figure 4.9 and Figure 4.12. Some useful ways of qualitatively analysing these plots include looking at how ‘pointed’ the end section is (which indicates how quickly the rate of nutation slows) and the concentration of the spirals along the ω2 axis (which indicates the initial damping of the system). These qualities are apparent in Figure 4.2 and Figure 4.5 where a high initial precession appears to have the effect of making ω1 approach zero faster. Given that ω1=θ, this makes sense as it corresponds to the physical tendency of the disk’s nutation to stop changing and remain almost upright while the disk is precessing. From Figure 4.5, there are no immediately clear conclusions about the effect of a high rate of revolution on the angular velocity. For the sake of clarity, these plots can be viewed from the top down (ω1 vs ω2) to more clearly show the relationships involved (as in Figure 5.1). It is clearer from this view that the angular velocity of the disk is far more sensitive to changes in the precession rate when the revolution rate is high. From Figure 4.9, it appears that the ω spirals become more concentrated as the drag in the xy-plane is increased (indicating that the initial damping of the system is larger) which is to be expected. However, changing the drag in the z-direction appears to have little to no effect on the evolution of the angular velocity (Figure 4.12).

| Figure 5.0a: ψhigh, ϕhigh | Figure 5.0b: ψhigh, ϕlow |
| --- | --- |
| Figure 5.0d: ψlow, ϕhigh | Figure 5.0c: ψlow, ϕlow |

Figure 5.1: ω1 vs ω2 with nonzero ψ0

## The evolution of the disk’s nutation

Figure 4.3 indicates that the initial nutation appears to be proportional to the frequency of oscillation of θ throughout the motion as well as the range of oscillation. This can be expected due to the fact that the disk will tend to align itself at a high nutation shortly after being spun at a low initial nutation. This also shows an interesting relationship between the initial nutation and the oscillatory frequency of θ where at a high initial nutation, an inverse relationship is observed however at a low initial nutation they appear to have a direct relationship. This is likely due to the fact that at a high θ0, the large precession rate will result in more rapid, stable motion of the disk however at a low θ0, the disk is less stable in its precession and its nutation tends to vary more dramatically but at a slower rate. Figure 4.10 and Figure 4.13 indicate that the same applies to the xy-plane drag where higher drag coefficients lead to a lower frequency of oscillation of θ. The same does not appear to apply to changes in Cz, in fact the opposite appears to be the case. A higher drag in the z-direction appears to lead to a larger oscillatory frequency. All forms of drag coefficient in all directions, however, expectedly lead to very fast damping of the amplitude of oscillation of θ.

## The approximated frictional model

The model used to take friction into account was an approximate answer to the question of how damping effects the motion of the disk. In reality there is air drag, surface friction between the disk and the surface, and potentially many other damping forces in effect. The surface friction is assumed to be zero (due to no slip) and the sum of all of the other damping forces are approximated as a drag induced torque (as in Equation 1.8) which is proportional to the inertia tensor of the disk and the square of the disk’s angular velocity and is moderated by drag coefficients. This is a simplistic model which obviously neglects the finer details of the material used, and the precise relationships involved as well as the calculations of the actual drag coefficients due to the sheer complexity involved being beyond the scope of this project (likely involving analysis of material properties and fluid dynamics to analyse the air drag).

## The error prone Lyapunov exponent

The Lyapunov exponent is a concept that applies primarily to exponential systems due to its characteristic equation deriving from an assumption of exponential growth. This inherently applies less accurately to systems which do not fit well to an exponential. This appears to be the case for the spinning disk system as the graphs in Figure 4.7 should ideally be close to perfectly linear but with varying slopes (if the distance between paths truly diverged exponentially). Looking at these figures and taking the regular oscillations as outliers, it can be seen that the increase is very roughly linear. The pattern however deviates frequently from this line making it very difficult to simply discount these frequent oscillations as just outliers, particularly when it comes to data analysis. This means that, as described above, the particular range of datapoints which one approximates to a line and uses to calculate λ is extremely subjective and error prone. The best analysis one can hope to achieve using Lyapunov exponents in the case of the spinning disk system involves looking at the graphs, selecting the best range available to approximate to a line based on reasonable criteria, and using those same criteria to select the range for all values of θ0. This does not solve the issue of the specific λ values being highly subjective and error prone, however, it does achieve a level of internal consistency between the values found for λ which is useful in modelling how the relative chaos of the spinning disk system changes with initial nutation. In order to more effectively measure the degree of chaos of the system, it is recommended that the Lyapunov method is somehow refined to better suit the spinning disk system or potentially for a different method other than Lyapunov exponents to be used.

## Chaotic nature of the system relative to the initial nutation

From Figure 4.14 looking at the bigger picture (5.14a), it is easy to see that the relative sensitivity of the system to the initial conditions is roughly the same for most values of θ0 until close to 90° at which point any very small changes in the initial nutation very significantly affect the system dramatically.

However, looking more closely at the behaviour throughout the full range of θ0 values (5.14b), it is clear that there is notable variation in the relative sensitivity, with local minima/maxima at the angles of 30°, 45° and 66°. It is tempting to look at the geometric significance of 60° angles fitting in more succinctly with 30° and 45°. The graph displays an interestingly sinusoidal behaviour, peaking at the exact centre of the range of possible θ0 values.

It is worth pointing out the omission of θ0<28° values. The reason for this was due to an inherent instability in the code which assumed one contact point between the surface and the disk. When the initial nutation is low enough combined with a low precession rate, the disk falls flat before it is able to enter a stable precession motion and thus the contact point is undefined as all points on the surface of the disk are contacting the opposing surface. This naturally leads to an indexing error in MATLAB. The exact nutation threshold below which the code cannot handle depends on the rate of precession. Intuitively if the disk is spun at a high enough rate even at a very low initial nutation, the disk will be able to reorient itself at a high angle of nutation (as described in section 6.3) before it falls flat thus allowing it to continue spinning. The necessary rate of precession to accomplish lower and lower initial nutations, however, quickly becomes unrealistically large and this generates its own problems. The best compromise found was to set the precession rate to 80 revolutions t-1 allowing nutations just lower than 30° to illustrate the symmetry of the behaviour about the 45° line without having an overly large precession rate.

## Why do these local extremes appear?

Looking at the graphs from Figure 4.15 to Figure 4.17, it could be ascertained that the retrograde pattern referenced by Jalali et al. [1] is related to the local extremes of sensitivity to initial perturbations. As the angle approaches θ=30° and θ=66° the retrograde appears to become more pronounced (with the motion entering retrograde earlier along the trajectory) whereas around θ≈45° the retrograde is less pronounced (with the trajectory appearing to “unravel” as θ is increased). It could potentially be that a low level of retrograde / a later retrograde may be somehow associated with high sensitivity of the system to changes in the initial conditions

An important point to note is the inconsistency of the time range used to produce the trajectory in 5.15a t=0 to t=6.99 as opposed to the time range used to produce every other graph t=0 to t=50. The reason for this exception relates to the error described in Section 6.6; the simulation failed after t=7 with an indexing error, implying that the disk had fallen flat after that time period. The reason why this became an issue now but not when measuring the Lyapunov exponent is that for the previous measurement, the time range of t=0 to t=0.05 was used to calculate the initial behaviour of lndt vs t. This made it appear as if the disk had settled into a stable motion but in reality, it simply took longer than 0.05t for the disk to fall flat. It could be that there is some proneness for the disk to fall flat up until the θ=45° mark at which point the disk is upright enough that it is more likely to continue spinning indefinitely due to the lack of surface friction. The other extreme would involve the disk being highly likely to fall flat θ=30° or highly likely to continue spinning in a stable motion θ=66°. This minimum is observed on the graph at θ=66°, however, it is also possible that this section of the graph (on either side of 45°) is symmetric and the “virtual minimum” is closer to 60°. This difference between this virtual minimum could be explained by observing the apparent tendency of the sensitivity to increase dramatically close to θ=90°. This tendency presumably has a negligible effect at low values of θ but the extent to which this tendency influences the behaviour of the overall sensitivity may increase slowly as θ increases until it becomes less and less negligible. As this influence grows to be on the order of the influence of the uncertainty relating to whether the disk stabilises or falls flat, the overall sensitivity shown in Figure 4.14 may increase slightly. This may explain the asymmetry in the behaviour of the sensitivity about 45° as the virtual minimum is being “pulled” towards the observed minimum by the increasing sensitivity near 90°.

# Conclusions

The spinning disk system is clearly one that is highly sensitive to the initial conditions of the Euler angles, their rates of change and the friction between the disk and the surface. The most affective of these variables are the initial nutation θ, the initial precession rate ϕ, the drag coefficients Cx, Cy, Cz and the initial revolution rate ψ. The initial nutation appears to largely determine the overall amount of orbiting or rocking motion of the disk throughout its orbit as well as which one is more dominant. The tendency of the nutation of the disk to oscillate at a very high frequency is observed more at higher initial nutations.

The initial precession rate is the largest determining factor when it comes to the overall distance travelled by the disk throughout its motion. A high precession rate also increases the oscillatory frequency of θ, so that ϕ∝νθ however this relationship is only observed at higher values of θ0; at lower values this relationship is inverted so that ϕ∝1νθ. Further analysis is needed to determine the precise relationship (e.g. inverse law, inverse square law, etc.).

The initial revolution rate dramatically increased the distance travelled by the centre of mass when it was the more dominant spin component acting on the disk, however, this effect could be cancelled out by increasing the precession rate to be of the same order. The system is also significantly more sensitive to changes in the precession rate when the revolution rate is also high.

Increasing the drag coefficients Cx and Cy led to significantly less initial orbiting of the centre of mass and also to a greater frequency of oscillation of θ Cx,Cy∝νθ. The xy-plane drag also appears to contribute far more to the overall damping of the disk motion than Cz does. The effects of increasing Cz appear to almost directly mirror those of increasing Cx and Cy whereas high Cz leads to significantly more initial orbiting, a lower oscillatory frequency of θ Cz∝1νθ and does not significantly impact the overall damping.

The relative sensitivity of the system to the initial nutation reaches local minima at angles of θ0=30°,66° and reaches a local maximum at θ0=45°; exactly halfway into the total range of θ. This appears to be related to the level of retrograde experienced by the centre of mass motion where a low/late retrograde effect is associated with high sensitivity to initial nutation. The sensitivity then appears to increase dramatically as the initial angle approaches θ=90°. It is possible that this increase has the effect of dragging the apparent minimum from its true location to the observed location of 66°

# Appendix

| θ | λ | θ | λ | θ | λ |
| --- | --- | --- | --- | --- | --- |
| 28 | 72.6450 | 49 | 136.7544 | 70 | 81.5838 |
| 29 | 70.6024 | 50 | 126.0812 | 71 | 83.7317 |
| 30 | 68.5794 | 51 | 122.2405 | 72 | 86.7674 |
| 31 | 72.0817 | 52 | 120.3661 | 73 | 88.5410 |
| 32 | 77.2120 | 53 | 113.6397 | 74 | 95.8728 |
| 33 | 85.5336 | 54 | 109.9399 | 75 | 99.0506 |
| 34 | 91.7740 | 55 | 107.6846 | 76 | 107.5283 |
| 35 | 98.0402 | 56 | 104.8207 | 77 | 109.9647 |
| 36 | 105.2798 | 57 | 97.6202 | 78 | 113.7994 |
| 37 | 107.0649 | 58 | 94.6429 | 79 | 127.4730 |
| 38 | 112.8604 | 59 | 93.2903 | 80 | 147.8338 |
| 39 | 122.9429 | 60 | 92.2087 | 81 | 164.9343 |
| 40 | 129.5178 | 61 | 89.2437 | 82 | 186.3686 |
| 41 | 136.8976 | 62 | 85.3296 | 83 | 205.6586 |
| 42 | 148.8340 | 63 | 81.9046 | 84 | 271.4705 |
| 43 | 152.5570 | 64 | 81.5086 | 85 | 275.8553 |
| 44 | 167.0119 | 65 | 78.3085 | 86 | 379.4066 |
| 45 | 173.2172 | 66 | 77.7820 | 87 | 631.5421 |
| 46 | 163.9791 | 67 | 77.8301 | 88 | 890.1528 |
| 47 | 153.0496 | 68 | 78.7930 | 89 | 2.3242e+03 |
| 48 | 143.2071 | 69 | 79.6218 |  |  |

Table 7.1: Recorded Lyapunov exponent for spinning disk system vs initial nutation

| θ | λrel | θ | λrel | θ | λrel |
| --- | --- | --- | --- | --- | --- |
| 28 | 1.059283 | 49 | 1.994103 | 70 | 1.189625 |
| 29 | 1.029499 | 50 | 1.83847 | 71 | 1.220945 |
| 30 | 1 | 51 | 1.782467 | 72 | 1.265211 |
| 31 | 1.051069 | 52 | 1.755135 | 73 | 1.291073 |
| 32 | 1.125877 | 53 | 1.657053 | 74 | 1.397982 |
| 33 | 1.24722 | 54 | 1.603104 | 75 | 1.44432 |
| 34 | 1.338215 | 55 | 1.570218 | 76 | 1.567939 |
| 35 | 1.429587 | 56 | 1.528458 | 77 | 1.603465 |
| 36 | 1.535152 | 57 | 1.423462 | 78 | 1.659382 |
| 37 | 1.561182 | 58 | 1.380049 | 79 | 1.858765 |
| 38 | 1.64569 | 59 | 1.360325 | 80 | 2.155659 |
| 39 | 1.792709 | 60 | 1.344554 | 81 | 2.405012 |
| 40 | 1.888582 | 61 | 1.301319 | 82 | 2.71756 |
| 41 | 1.996191 | 62 | 1.244245 | 83 | 2.998839 |
| 42 | 2.170244 | 63 | 1.194303 | 84 | 3.958485 |
| 43 | 2.224531 | 64 | 1.188529 | 85 | 4.022422 |
| 44 | 2.435307 | 65 | 1.141866 | 86 | 5.53237 |
| 45 | 2.525791 | 66 | 1.134189 | 87 | 9.208918 |
| 46 | 2.391084 | 67 | 1.13489 | 88 | 12.97989 |
| 47 | 2.231714 | 68 | 1.148931 | 89 | 33.89064 |
| 48 | 2.088194 | 69 | 1.161016 |  |  |

Table 7.2: Normalised Lyapunov exponent for spinning disk system vs initial nutation

# Contribution Checklist

|  | Myself | Others | Comments |
| --- | --- | --- | --- |
| Research and Literature Review |  |  |  |
| Review of research papers on the topic | 80% | 20% | Supervisor provided copies of two research papers on the dynamics of rolling rings. |
| Research into mechanics of rigid bodies | 90% | 10% | Supervisor recommended Classical Dynamics of Particles and Systems, Chapter 11. |
| Code Design and Preparation |  |  |  |
| EqOfMotionModel.m | 10% | 90% | Supervisor wrote the model to simulate the motion and I adjusted the code to model λ |
| LyapunovExponentPlotter.m | 100% | 0% | I wrote a separate code file to record and plot λ |
| Debugging |  |  |  |
| Debugging EqOfMotionModel.m | 50% | 50% | I debugged some initial issues relating to the variation of the initial position of the COM due to changes to θ0 |
| Inability to lower θ0 past a certain threshold | 50% | 50% | My supervisor clarified that this issue was due to an inherent instability with the models determination of the contact point in the code. |
| Analysis |  |  |  |
| System sensitivity analysis | 100% | 0% | I generated graphs to visualise the system behaviour and analysed the chaotic nature of the system. |
| Discussion |  |  |  |
| Discussion of results and conclusions | 100% | 0% | I qualitatively described the significance of the graphs generated and the potential errors involved. |

# References

# [1] Jalali, M.A., Sarebangholi, M.S., Alam, M.-R. (2019) Terminal Retrograde Turn of Rolling Rings [online], Physical Review, available: https://journals.aps.org/pre/abstract/10.1103/PhysRevE.92.032913 [accessed 13 Oct 2019].

# [2] Leine, R.I. (2009) ‘Experimental and theoretical investigation of the energy dissipation of a rolling disk during its final stage of motion’, Archive of Applied Mechanics, 79(11), 1063–1082.

# [3] Ma, D., Liu, C., Zhao, Z., Zhang, H. (2014) ‘Rolling friction and energy dissipation in a spinning disc’, Proceedings of the Royal Society A: Mathematical, Physical and Engineering Sciences, 470(2169), 20140191.

# [4] Press, W.H., Tevkolsky, S.A., Vetterling, W.T., Flannery, B.P. (2007) Numerical Recipes: The Art of Scientific Computing, 3rd ed, Cambridge University Press.

# [5] Thornton, S.T., Marion, J.B. (2004) Classical Dynamics of Particles and Systems, 5th ed, Cengage.

# [6] Walsh, P., Clancy, I. (2020) The Dynamics of Spinning Disks on Frictional Surfaces [online], University Of Limerick, available: https://github.com/PatrickWalsh90598/The-dynamics-of-spinning-disks-on-frictional-surfaces.